Ramkishore Dhakad                                                                                                           ramdhakad20@iitk.ac.in

Solid Mechanics and Design

Department of Mechanical Engineering

Indian Institute of Technology Kanpur, Uttar Pradesh, India.

Contents:

Introduction

Functions:

- Primary Function

- Secondary function

- Tertiary Function

Objectives

Constraints

Free variables

Material Index between Density and Properties of material:

- Strength and Density

- Young’s Modulus and Density

- Shear Modulus and Density

- Fracture Toughness and Density

- Fatigue strength (endurance strength) and density

- Toughness (U):

- Thermal expansion coefficient and density

- Elongation of Vertical shaft:

- Machinability and Density

10.Hardness and Density

Material Index between any two Properties of material:

- Fracture Toughness and Young’s modulus:

- Mechanical loss coefficient (tanδ) and Young’s Modulus

- Young’s Modulus and Strength:

- Specific Strength and Specific Modulus:

- Shear modulus and shear strength:

- Machinability:

- Wear rate constant and Hardness:

- Thermal conductivity and Heat storage capacity:

- Mechanical loss coefficient (tanδ) and Young’s Modulus and Density

- Relative cost and material class:

- Modulus and relative cost:

Conclusion

References

Introduction:

Defination:

Material Selection:

A very wide range of material is available to us.

Selecting a material for particular device or component is a very tedious job.

To select material for any component, first we need to a detailed analysis of that component.

The interaction between function, material, shape, and process lies at the heart of the material selection process.

In this Document Author tried to select the best possible material for the Shaft.

Shaft:

A shaft is a rotating machine element, usually circular in cross section, which is used to transmit power from one part to another, or from a machine which produces power to a machine which absorbs power [3].

Types [3]:

They are mainly classified into two types.

Transmission shafts are used to transmit power between the source and the machine absorbing power; e.g. counter shafts and line shafts.

Machine shafts are the integral part of the machine itself; e.g. crankshaft.

Axle shaft.

Spindle shaft.

The following stresses are induced in the shafts.

Shear stresses due to the transmission of torque (due to torsional load).

Bending stresses (tensile or compressive) due to the forces acting upon the machine elements like gears and pulleys as well as the self-weight of the shaft [1].

Stresses due to combined torsional and bending loads.

Primary Function:

Yield Strength of material:

Primary function of the shaft is to transmit torque and carry power transmission element because of torque torsional stresses developed and because of the weight of shaft and power transmission element bending stresses developed in the shaft. Hence shaft is subjected to combined loading. To withstand these stresses shaft material must have higher yield strength.

shaft is generally made of ductile material and hence Tressa or Von-Misses criteria can be used for design of shaft.

According to the Tressa theory the shear strength and tensile strength is related as follows:

τy=0.5σy

Von-Misses criteria the shear strength and tensile strength is related as follows:

τy=σy3

Hence higher the tensile yield strength of material higher will be the yield strength in shear.

Bending stiffness

Bending stiffness is a measure of resistance to bending deformation of a beam. It is measure of amount of a beam will deform under a given load. Higher the bending stiffness laser will be the deformation.

shaft is subjected to the radial loading perpendicular to the axis of the shaft and hence bending deformation of the shaft takes place.

in general bending stiffness is given as follows:

Sb=EIL

deflection of beam with both ends fixed and a point load acting at the center is given by:

δ=WL3192EI

and from this equation we can see that deflection is inversely proportional to the bending stiffness.

Torsional stiffness

It is defined as the torque required to produce a twist of unit radian.

The torsional stiffness is given by:

St=Tθ

Using torsional equation:

TJ=τR=GθL
St=GJL
θ=TLGJ

Shaft is subjected to different amount of torque at different cross section along the length and hence there is possibility of twisting of the shaft along the length the twist in the shaft should be minimum during the working period.

Higher the torsional stiffness laser will be the angular twist for given amount of torque on shaft.

Fracture Toughness:

Fracture toughness is a measure of resistance offered by material against the propagation of crack. Higher the fracture toughness of material laser will be the chance of propagation of crack inside or on the surface of material.

Depending upon the manufacturing process of shaft like casting, welding or machining there is a possibility of presence of cracks inside or on the surface of the components.

During casting because of the gas defect and in welding because of the non-uniform heating and cooling of component can cause crack can generate inside the component.

The chances of propagation of crack on the shaft is verify because the shaft is subjected to the completely reversed loading in which the mean stress is zero (σmean=0 ) and only amplitude stress is exist. Therefore, shaft material with higher fracture toughness is needed.

KIC=Yσcπc

Y: a geometric factor

E: Young's modulus

σc: Tensile stress at which crack will propagate

KIC : fracture toughness

2c: Crack length

Toughness:

It is very important property when component subjected to the impact loading or any kind of jerk. A tough material means material is having high strength and high ductility.

Toughness is defined as the area under stress strain curve.

It is also defined as the amount of energy per unit volume that a material can absorb before rupturing.

shaft used in some places like in Automobile, wind turbine sometime subjected to sudden loading and hence to avoid failure of the shaft, material of the shaft must be tough.

Mathematically toughness can be defined as:

energy volume=0εfσ dε

ε:Strain
σ:Stress
εf:strain upon fracture

Density:

Higher the density of material higher will be the mass of shaft. It is preferred to have less density of material. Specially in space application like the shaft used in compressor and turbine this is a very important parameter.

Secondary Function

Fatigue strength (Endurance strength):

It is defined as the highest stress that material can withstand for given number of cycles without breaking of shaft.

Shaft is subjected to cyclic loading that is magnitude of stress at a point change from positive stress to zero and then it becomes negative stress of same magnitude, This happens in every single rotation of the shaft.

Since the loading on the shaft is dynamic and hence the design of the shaft is done using endurance strength of material.

For safest design Soderberg criteria is used:

σmσy+KfσVσe=1N

For σm=0

KfσV=σeN

σm:mean stress σmax+σmin2
σy:yield strength of material
σe:Endurance strength
Kf:Fatigue stress concentration factor
 N:Factor of safety
σV:stress amplitude σmax-σmin2

Endurance strength is defined is the highest value of completely reversed bending stress that a material can withstand for infinite number of cycles without any sign of crack. Generally, 10^7 cycles are considered as sufficient to calculate fatigue strength.

Mechanical loss coefficient:

The mechanical loss-coefficient also called damping factor measures the degree to which a material dissipates vibrational energy.

In some special application of shaft like in racing car, shaft used in machine tool in which vibration can affect the tolerance of component during machine it becomes important to consider loss coefficient.

If a material subjected to elastic loading of σmax ,then it stores elastic energy per unit volume U:

U=0σmax σ dε= 12 σmax2E

If the material is then unloaded, it dissipates an energy:

ΔU=∮ σdε

The loss coefficient is

η=ΔU2πU

The value of η usually depends on the frequency of cycling. Damping can also measure by

- specific damping capacity

D=ΔU/U

- The logarithmic decrement,

∆ =ln⁡(x2x1)

Where x1 and x2 are successive amplitudes of natural vibrations.

- The phase-lag, δ, between stress and strain, and the " Q "-factor or resonance factor, Q.

When damping is small (η<0.01) these measures are related by

η=D2π=Δπ=tanδ=1Q

For large damping above relations are not valid. Damping for case of shaft design is considered small while for case of spring Design it considered large.

Loss Tangent is defined as:

Loss tangent=Loss Modulusstorage modulus

Storage modulus:

G'ω=σ0cosδγ0

Loss Modulus:

G''ω= σ0sinδγ0
tanδ=G''ωGin'ω 
δ=0 Hookean Elastic solid
0<δ<90∘. Viscaclastic paterual
δ=90∘ viscousliquid.
σ=stress  γ= strain

Machinability:

Machinability is not a property of material. It is a process parameter. In the selection of material, it is also important to consider the process parameter so that we can machine component easily in a desired shape.

Machinability refers to the ease with which a material can be machined to an acceptable surface finish

machinability is measured in terms of following parameters:

- Tool wear during machining.

- Form of chip produced during machining continuous or discontinuous chip),Ductile material produces continuous chip while the brittle material produces discontinuous chip.

- Surface finish

- Cutting force required.

Note: Most of the time ductile material having better machinability than brittle material.

Linear coefficient of thermal expansion:

linear coefficient of thermal expansion is defined as the change in length of material per unit rise in temperature.

It measures the extent to which material expands upon heating.

when shaft used in high temperature application such as in steam turbine gas turbine it becomes very important to consider coefficient of thermal expansion during selection of materials.

Hardness:

hardness is a quick measure of strength of material. hardness can be defined as resistance to wear, scratch or indentation. it is also defined as resistance to localized plastic deformation of material

hardness is measured by prancing indenter on the surface of material and then calculating the indentation on surface.

Based on different shape of indenter we have following different hardness numbers:

- Brinell hardness number

- Rockwell hardness number

- Vicker hardness number

H=Constantσf

The value of constant is very from material to material. For ferrous materials:

H=3σf

It has units of MPa. Hardness is most usually reported in other units, Vickers Hv scale with units of kg/mm2. It is related to H by:

HV=H10

Cost:

Relative cost=cost of materialkg*density of materialcost of mild steelkg*density of mild steel

Tertiary Function:

Coefficient of thermal expansion:

ΔL=αLLΔT

αL:linear Coefficient of thermal expansion

Thermal Diffusivity:

T is the ratio of thermal conductivity

Objectives:

Primary Objectives:

- Maximize the strength of shaft.

- Maximize specific strength.

- Maximize specific modulus

- Minimize the weight

- Minimize the deflection of shaft due to various kind of loading on the shaft

- Maximize the bending stiffness of the shaft.

- Minimize the torsional rigidity of the Shaft

- Minimize the angle of Twist of shaft due to Torque acting on it.

Secondary Objectives:

- Maximize the fracture toughness of shaft.

- Maximize the toughness of shaft.

- Maximize the linear coefficient of thermal expansion

- minimize the cost

Tertiary Objectives:

- Selection of materials with higher machinability.

- Minimizing the bending strain of shaft.

- Maximize the mechanical loss coefficient

- Maximize the thermal diffusivity of shaft material

Constraints:

- Length L specified

- Bending Stiffness: must not deflect too much under design loads

- Torsional Stiffness: must not twist too much under design Torque

- Strength: must not fail under design load (σy>300 MPa)

- Fracture toughness: KIC>15 MPa.m0.5

Free Variable:

Selection of Material.

Radius of the shaft.

Note: In most of the chart the young’s modulus taken > 150Gpa

Material Indexes using material property/Characteristics and Density:

- Strength and Density:

Torsion Equation:
TJ=τr=GθL
τ=TrJ
 polar moment of inertia J=π2r4 
τ=2Tπr3 
τfN=2Tπr3
 m=πr2Lρ
 r=mπLρ
τfN=2TπmπLρ3=2TπL3ρ3m3
m=(2NT)23π13L(ρτf23)

Thus, performance index given by
P1=(τf23ρ)maximize

| Material | σf | ρ | (τf23ρ)maximize |
| --- | --- | --- | --- |
| Carbon fiber (HS carbon) | 3870 | 1.82 | 135.4 |
| Carbon fiber (Hm carbon) | 2400 | 1.82 | 98.5 |
| Glass fiber ( C - glass) | 3250 | 2.57 | 85.4 |
| CFRP (epoxy matrix) | 760 | 1.55 | 53.7 |
| Titanium alloys | 949 | 4.6 | 21.0 |
| High carbon steel | 680 | 7.85 | 9.9 |
| Medium carbon steel | 591 | 7.8 | 9.0 |
| Stainless steel | 541 | 7.74 | 8.6 |
| Nickle alloy | 377 | 8.772 | 5.9 |

Taking log on both side of performance index we get equation of line with slope 1.5.

a horizontal line at Young's modulus 210GPa is drawn.

material above both lines will be the good material for this index.

from chart Carbon fibre (high strength) is having maximum performance index while Nickel alloys having lowest performance index

- Young’s Modulus and Density:

Bending Equation:

MI=σY=ER

Bending Stiffness:

Sb=EIL
Combining above two equations
Sb=EΓL=Eπγ44L
r=mπLρ
Sb=Eπ4LmπLρ2
m2=4SbπL2ρ2E
m=4SbπL2ρE12

Thus, performance index given by
P2=(E1/2ρ)Maximize

| material | E | ρ | (E1/2ρ)Maximize |
| --- | --- | --- | --- |
| Carbon fiber (HM cabon) | 380 | 1.82 | 10.71 |
| Carbon fiber (HS cabon) | 242 | 1.82 | 8.55 |
| High carbon steel | 207 | 7.85 | 1.83 |
| Stainless steel | 200 | 7.74 | 1.83 |
| Nickle based super alloy | 210 | 8.27 | 1.75 |
| Nickle chromium alloy | 210 | 8.4 | 1.73 |

Taking log on both side of performance index we get equation of line with slope 2.

a horizontal line at Young's modulus 200GPa is drawn.

material above both line will be the good material for this index.

from chart abain Carbon fibre ( high strength) is having maximum performance index while Nickel alloys having lowest performance index

- Shear Modulus and Density:

Torsional Equation:
TJ=τr=GθL
Torsional stiffness:

St=Tθ= GJL
St=Gπr42L
St=Gπ2LmπLρ2
m2=2StπL2ρ2G
m=2StπL2ρG12

Thus, performance index given by
P3=(G12ρ)maximize

| Material | G | ρ | (G12ρ)maximize |
| --- | --- | --- | --- |
| Carbon fibre (HM carbon) | 170 | 1.82 | 7.2 |
| Carbon fibre (HS carbon) | 105 | 1.82 | 5.6 |
| CFRP (epoxy matrix) | 60 | 1.55 | 5.0 |
| Titanium alloys | 42.4 | 4.6 | 1.4 |
| Stainless steel | 77.9 | 7.74 | 1.1 |
| Nickle chromium alloy | 76.8 | 8.4 | 1.0 |
| Tungsten alloy | 130 | 17.7 | 0.6 |

Taking log on both side of performance index we get equation of line with slope 2.

a horizontal line at Shear modulus 75GPa (shear modulus of steel) is drawn.

material above both line will be the good material for this index.

from chart again Carbon fibre ( high strength) is having maximum performance index while Tungsten alloy having lowest performance index

- Fracture Toughness and Density:

KIC=Y σcπa                                (1)

σcN= 4Mπr3                                           (2)

m= πr2Lρ                                         3

r= mπLρ                                        (4)

Then equation (2) becomes,

σcN= 4Mπ[mπLρ]3/2

σc=4MNπL3ρ3m3             (5)

From equation (1),

KIC=Y.4MNπL3ρ3m3.πa            (6)

m= 4YMN23π23a13.(ρKIC2/3)

4YMN23π23a13  is constant.

Thus, performance index given by

P4= KIC2/3ρmaximum

| material | KIC | ρ | KIC2/3ρ |
| --- | --- | --- | --- |
| Nickle based super alloy | 179 | 8.27 | 3874 |
| Stainless steel | 88.4 | 7.74 | 1010 |
| Titanium alloys | 62 | 4.6 | 836 |
| Low alloy steel | 56.4 | 7.8 | 408 |
| Non age hardning alluminium alloy | 31.6 | 2.66 | 375 |
| High carbon steel | 49.8 | 7.85 | 316 |
| cast Al Alloy | 24.2 | 2.71 | 216 |
| CFRP (epoxy matrix) | 11.1 | 1.55 | 79 |

Taking log on both side of performance index we get equation of line with slope 1.5.

a horizontal line at Fracture Toughness (>15Mpa.m^0.5) is drawn.

material above both lines will be the good material for this index.

from chart Nickle alloy is having maximum performance index while CFRP composite is having lowest performance index

- Fatigue strength (endurance strength) and density:

For safest design Soderberg criteria is used:

σmσy+KfσVσe=1N

For σm=0

KfσV=σeN

σm:mean stress σmax+σmin2
σy:yield strength of material
σe:Endurance strength
Kf:Fatigue stress concentration factor
 N:Factor of safety
σV:stress amplitude σmax-σmin2

M: Bending moment

From shear stress formula,

τaN*r=(Mmax-Mmin)2J=GδθL

Where, Mf (equivalent fatigue torque) = KbM2+KtT22

Kb & Kt are combined shock and fatigue factor applied to bending moment and torque respectively

J (Polar moment of inertia) = π2r4 &  r=mπLρ

m=N(Mmax-Mmin)23π13Lρτe23

N(Mmax-Mmin)23π13L is constant

Thus, performance index given by

P5=τe23ρmaximize

| Material | σe | ρ | σe23ρmaximize |
| --- | --- | --- | --- |
| Carbon fibre (HS carbon) | 3950 | 1.82 | 137.3 |
| Glass fibre (C - glass) | 2920 | 2.57 | 79.5 |
| Titanium alloys | 603 | 4.6 | 15.5 |
| Age hardening wrought AL Alloy | 148 | 2.75 | 10.2 |
| Stainless steel | 372 | 7.74 | 6.7 |
| Nickel based super alloy | 405 | 8.27 | 6.6 |
| CFRP (epoxy matrix) | 212 | 1.55 | 22.9 |
| Carbon fibre (Hm carbon) | 2120 | 1.82 | 90.7 |

Taking log on both side of performance index we get equation of line with slope 1.5.

a horizontal line at endurance strength (>100 Mpa) is drawn.

material above both lines will be the good material for this index.

from chart High strength carbon fiber is having maximum performance index while high modulus carbon fibre is having lowest performance index

- Toughness (U):

U=KIC 2E

Using equation (6) of fracture toughness from last derivation:
 KIC=4YMNπL3 ρ3 m3 πa
U=(4YMN)2π2aL3Eρ3m3
m3=(4YmN)2π2aL3ρ3EU
m=(4YmN)23π2aL313ρE13U13

Thus, performance index given by
P6=(EU)13ρ  maximize.

Taking log on both side of performance index we get equation of line with slope 1.5.

a horizontal line at endurance strength (>100 Mpa) is drawn.

material above both lines will be the good material for this index.

from chart High strength carbon fiber is having maximum performance index while high modulus carbon fibre is having lowest performance index

- Thermal expansion coefficient and density:

ΔL=αLLΔT

m=πr2Lρ

ΔL=αLmΔTπr2ρ

m=ΔLπr2ρΔTαL   -(1)

Shaft is of fixed length thus temperature rise will increase the stress:

ΔL=σL/E -(2)

Using (1) and (2)

m=σLπr2ρΔTEαL

πr2σLΔT is constant

Thus, performance index given by

P7=EαLρmaximize

| material | EαL | ρ | EαLρ |
| --- | --- | --- | --- |
| Age hardening wrought Al Alloy | 1820 | 2.75 | 662 |
| Stainless steel | 2670 | 7.74 | 345 |
| High carbon steel | 2530 | 7.85 | 322 |
| Low alloy steel | 2450 | 7.8 | 314 |
| Nickle based super alloy | 2570 | 8.27 | 311 |
| CFRP (epoxy matrix) | 429 | 1.55 | 277 |
| Titanium alloys | 1060 | 4.6 | 230 |
| Tungsten alloy | 1670 | 17.7 | 94 |

Taking log on both side of performance index we get equation of line with slope 1.

material on right of vertical line and above horizontal will be the good material for this index.

from chart Age hardening wrought Al Alloy is having maximum performance index while Tungsten alloy is having lowest performance index.

- Elongation of Vertical shaft:

Elongation of shaft due to self-weight,

δl= WL2AE                           (1)

Where, W is self-weight of the shaft.

W=mg=ρALg            (2)

Put the value from equation (2) in (1),

δl= ρALgL2AE

E= gL22.ρδl

δl= gL22.ρE

gL22 is constant.

Thus, performance index given by

P8= Eρmaximum

| material | E | ρ | Eρ |
| --- | --- | --- | --- |
| Carbon fibre (HM carbon) | 380 | 1.82 | 208.8 |
| Carbon fibre (HS carbon) | 242 | 1.82 | 133.0 |
| CFRP (epoxy matrix) | 150 | 1.55 | 96.8 |
| High carbon steel | 207 | 7.85 | 26.4 |
| Stainless steel | 200 | 7.74 | 25.8 |
| Titanium alloys | 115 | 4.6 | 25.0 |
| Nickle chromium alloy | 210 | 8.4 | 25.0 |

Taking log on both side of performance index we get equation of line with slope 1.

material on left of vertical line and above horizontal line (E>100GPa) will be the good material for this index.

from chart Carbon fibre (HM carbon) is having maximum performance index while Nickle chromium alloy is having lowest performance index.

- Machinability and Density:

machinability is not a property of material. It is a process parameter. In the selection of material, it is also important to consider the process parameter so that we can machine component easily in a desired shape.

There is no direct empirical relation between the machinability and the density of the material. We can consider two separate material indexes. we need to maximize machinability and minimize the density.

Pa=Machinability      Maximize
Pb=Density      Minimize

Thus, performance index given by
P9=PaPb=MachinabilityDensity
P9 need to be maximixe

Note: For the Machinability of the material, rating is given from one to five, one means very difficult to machining and five means having Excellent machining.

Based on this machinability index is calculated for different materials.

| Material | Machinability Index | Density | MachinabilityDensity |
| --- | --- | --- | --- |
| Aluminium Alloy | 4.47 | 2.71 | 1.6494 |
| CFRP (epoxy matrix) | 1.73 | 1.55 | 1.1161 |
| High Carbon Steel | 3.46 | 7.8 | 0.4436 |
| Medium Carbon Steel | 3.46 | 7.8 | 0.4436 |
| Low Alloy Steel | 3.46 | 7.8 | 0.4436 |
| Titanium alloys | 1.73 | 4.61 | 0.3753 |
| Nickle Alloy | 3 | 8.32 | 0.3606 |
| Stainless steel | 2.45 | 7.8 | 0.3141 |
| Tungsten Alloys | 1.41 | 17.7 | 0.0797 |

Taking log on both side of performance index we get equation of line with slope 1.

material on left of vertical line and above horizontal line (Machinability index>1.75 ) will be the good material for this index.

from chart Aluminium Alloy is having maximum performance index while Tungsten Alloys is having lowest performance index.

- Hardness and Density:

Hardness is a quick measure of strength of material. hardness can be defined as resistance to wear, scratch or indentation. it is also defined as resistance to localized plastic deformation of material.

We need to maximize the hardness and minimize the density.

Pa=Hardness      Maximize
Pb=Density      Minimize

Thus, performance index given by
P10=PaPb=HardnessDensity
P10 need to be maximixe

| Material | Hardness | density | HardnessDensity |
| --- | --- | --- | --- |
| Carbon fibre (HM carbon) | 731 | 1.82 | 401.6 |
| Carbon fibre (HS carbon) | 731 | 1.82 | 401.6 |
| Titanium alloys | 319 | 4.6 | 69.3 |
| Low alloy steel | 333 | 7.8 | 42.7 |
| High carbon steel | 322 | 7.85 | 41.0 |
| Medium carbon steel | 276 | 7.8 | 35.4 |
| Nickle based super alloy | 253 | 8.27 | 30.6 |

Taking log on both side of performance index we get equation of line with slope 1.

material on left of vertical line and above horizontal line (H>210) will be the good material for this index.

from chart Carbon fibre (HM carbon) is having maximum performance index while Nickle based super alloy is having lowest performance index.

Material Indexes using Two material property/Characteristics:

Fracture Toughness and Young’s modulus:

KIC=Y σcπa

Toughness of material in terms of the fracture toughness, young’s modulus and poison ratio is expressed as follows [1]

U=KIC 2E(1+ϑ)

If we want to maximize U ,we can Maximize the right hand side of index.

P1=(KIC 2E(1+ϑ))maximize

logKIC=2logE1+ϑ+2logU

| Material | KIC | E(1+ϑ) | (KIC 2E(1+ϑ))maximize |
| --- | --- | --- | --- |
| Nickle based Super alloy | 179 | 274 | 117 |
| Tungsten Alloys | 134 | 433 | 41 |
| Nickle chromium Alloy | 93.8 | 275 | 32 |
| Stainless steel | 88.4 | 255 | 31 |
| Titanium Alloy | 62 | 156 | 25 |
| Low Alloy Steel | 56.4 | 263 | 12 |
| Age hardening wrought Al Alloy | 32.5 | 96 | 11 |
| High Carbon Steel | 49.8 | 267 | 9 |
| Medium carbon steel | 48 | 270 | 9 |
| CFRP epoxy matrix | 11.1 | 133 | 1 |

Taking log on both side of performance index we get equation of line with slope .

material on left of vertical line and above horizontal line (Fracture toughness>15MPa.m^0.5) will be the good material for this index.

from chart Nickle based Super alloy is having maximum performance index while CFRP epoxy matrix is having lowest performance index.

Mechanical loss coefficient (tanδ) and Young’s Modulus:

The mechanical loss-coefficient also called damping factor measures the degree to which a material dissipates vibrational energy.

If a material subjected to elastic loading of σmax ,then it stores elastic energy per unit volume U:

U=0σmax σ dε= 12 σmax2E

If the material is then unloaded, it dissipates an energy:

ΔU=∮ σdε

The loss coefficient is

η=ΔU2πU

The value of η usually depends on the frequency of cycling. Damping can also measure by

- specific damping capacity

D=ΔU/U

- The logarithmic decrement,

∆ =ln⁡(x2x1)

Where x1 and x2 are successive amplitudes of natural vibrations.

- The phase-lag, δ, between stress and strain, and the " Q "-factor or resonance factor, Q.

When damping is small (η<0.01) all of the above related by:

η=D2π=Δπ=tanδ=1Q

For large damping above relations are not valid. Damping for case of shaft design is considered small while for case of spring Design it considered large.

Δ= 2πη1-η2
for  η<0.01          1-η2 ≈1

Δ =πtanδ=2πη1-η2Δ =πtanδ=2πη=2πCCc=2πC2km =2πC2mgδ⋅m=2πC2mgδ =C11mWL348EI=C21m 1E 1r2Δ =C2r2mE =C3Em=πtanδΔ =πtanδ

m =C3(1E tanδ)minimize

P2=(E tanδ)maximize

logE=-2log tanδ+logP2

| Material | E | tanδ | (E tanδ)maximize |
| --- | --- | --- | --- |
| CFRP epoxy matrix | 102 | 21.5 | 217 |
| Nickle chromium Alloy | 210 | 8.94 | 130 |
| Titanium Alloy | 115 | 10 | 107 |
| Stainless steel | 200 | 6.1 | 86 |
| Medium carbon steel | 210 | 5.89 | 85 |
| Low Alloy Steel | 205 | 4.27 | 61 |
| Nickle based Super alloy | 210 | 3.79 | 55 |
| Tungsten Alloys | 339 | 1.73 | 32 |

Taking log on both side of performance index we get equation of line with slope .

material on left of vertical line and above horizontal line (Fracture toughness>15MPa.m^0.5) will be the good material for this index.

from chart CFRP epoxy matrix is having maximum performance index while Tungsten Alloys is having lowest performance index.

Young’s Modulus and Strength:

Bending strain:

Bending equation

MI=σy=ER

For linear elastic isotropic material

E=σε
ε=σE

From bending equation

yR=σE=εb
εb=1NσyEminimize
P3=(Eσy)maximize

| Material | E | σy | (Eσy)maximize |
| --- | --- | --- | --- |
| Nickle chromium Alloy | 210 | 410 | 0.5122 |
| Tungsten Alloys | 339 | 776 | 0.4369 |
| Nickle based Super alloy | 210 | 496 | 0.4234 |
| Stainless steel | 200 | 541 | 0.3697 |
| Medium carbon steel | 210 | 591 | 0.3553 |
| High Carbon Steel | 207 | 680 | 0.3044 |
| Low Alloy Steel | 205 | 866 | 0.2367 |
| Carbon fiber (HM Carbon) | 380 | 2400 | 0.1583 |
| CFRP epoxy matrix | 102 | 760 | 0.1342 |
| Titanium Alloy | 115 | 949 | 0.1212 |
| Carbon fiber (HS Carbon) | 242 | 3870 | 0.0625 |

Specific Strength and Specific Modulus:

Bending equation

MI=σy=ER

For linear elastic isotropic material

E=σε
ε=σE

From bending equation

yR=σE=εb
εb=1NσyEminimize

Young’s modulus and the yield strength of material is divided by density individually:

P4=(Eρσyρ)maximize

| Material | Eρ | σyρ | (Eρσyρ)maximize |
| --- | --- | --- | --- |
| Nickle chromium Alloy | 0.025 | 0.0488 | 0.5123 |
| Tungsten Alloys | 0.0191 | 0.0438 | 0.4361 |
| Nickle based Super alloy | 0.0254 | 0.0599 | 0.4240 |
| Stainless steel | 0.0258 | 0.0699 | 0.3691 |
| Medium carbon steel | 0.0269 | 0.0758 | 0.3549 |
| High Carbon Steel | 0.0264 | 0.0866 | 0.3048 |
| Low Carbon Steel | 0.0263 | 0.111 | 0.2369 |
| Age hardening wrought Al Alloy | 0.0261 | 0.129 | 0.2023 |
| CFRP epoxy matrix | 0.0657 | 0.491 | 0.1338 |
| Titanium Alloy | 0.025 | 0.206 | 0.1214 |
| Carbon fibre (HS Carbon) | 0.133 | 2.13 | 0.0624 |

Taking log on both side of performance index we get equation of line with slope 1.

from chart Nickle chromium Alloy is having maximum performance index while Carbon fibre (HS Carbon)is having lowest performance index.

Shear modulus and shear strength:

Angle of twist of shaft:

Torsion Equation:
TJ=τr=GθL

Angle of twist:

θ=τLGr
θ=Lr1NτyGminimize
P4=(Gτy)maximize
Using von mises criterion:

τy=σy3
P5=(Gσy)maximize

| MAterial | G | σy | (Gσy)maximize |
| --- | --- | --- | --- |
| Nickle chromium Alloy | 76.8 | 410 | 0.1873 |
| Tungsten Alloys | 130 | 776 | 0.1675 |
| Nickle based Super alloy | 79.2 | 496 | 0.1597 |
| Stainless steel | 77.9 | 541 | 0.1440 |
| Medium carbon steel | 80.9 | 591 | 0.1369 |
| High Carbon Steel | 80.4 | 680 | 0.1182 |
| Low Carbon Steel | 80.9 | 866 | 0.0934 |
| CFRP epoxy matrix | 41 | 760 | 0.0539 |
| Titanium Alloy | 42.4 | 949 | 0.0447 |
| Carbon fibre (HS Carbon) | 105 | 3870 | 0.0271 |

Taking log on both side of performance index we get equation of line with slope 1.

from chart Nickle chromium Alloy is having maximum performance index while Carbon fibre (HS Carbon)is having lowest performance index.

Machinability:

For the Machinability of the material, rating is given from one to five, one means very difficult to machining and five means having Excellent machining.

Based on this machinability index is calculated for different materials.

P6=Machinability index

| Material | Machinability Index |
| --- | --- |
| Aluminium Alloy, wrought | 4.47 |
| High Carbon Steel | 3.46 |
| Medium Carbon Steel | 3.46 |
| Low Alloy Steel | 3.46 |
| Nickle Alloy | 3 |
| Stainless steel | 2.45 |
| CFRP (epoxy matrix) | 1.73 |
| Titanium alloys | 1.73 |
| Tungsten Alloys | 1.41 |

from chart Aluminium Alloy is having maximum performance index while Tungsten Alloys  is having lowest performance index

Wear rate constant and Hardness:

When one surface slides over another surface they wear. During wear, from both surfaces material is lost.

Wear rate is defined as:

W=Volume of material removed from contact surfaceDistance slid

It has unit of (m2)

Specific wear rate:

Ω=WAn

An:Nominal area

Multiply and divide by Fn nominal force  and bearing pressure P=FnAn

ΩP=WFn=Ka

Where Ka is wear constant

It measures relative wear with respect to bearing pressure higher the wear constant more we be the wear for given bearing pressure.

maximum bearing pressure can be related to Hardness as follows:

Pmax=CH

C is a constant

Equation of Ka multiply and divide by Pmax we got:

Ω=C(PPmax)(KaH)

lowest value of the product (KaH) is required for best wear resistance surface.

P7=(KaH)minimize

logKa=-logH+logP6

Note: It was difficult for me to get exact values of wear rate constant for each material and hence table is not given for this case.

But from ashby chart we can easily select the material which is having lower value of KaH

In this chart rating is given to different material based on their wear resistance.

Thermal conductivity and Heat storage capacity:

Maximize the thermal diffusivity of material

α=KρCpmaximize

P8=KρCpmaximize

where

α:Thermal Diffusivity

K:Thermal conductivity

ρCp:Heat storage capacity.

| Material | K | ρCp | KρCpmaximize |
| --- | --- | --- | --- |
| Carbon fiber (HM Carbon) | 126 | 1.29 | 98 |
| Age hardening wrought Al Alloy | 158 | 2.58 | 61 |
| Tungsten Alloys | 90.9 | 2.39 | 38 |
| Medium carbon steel | 50.4 | 3.68 | 14 |
| High Carbon Steel | 49.9 | 3.72 | 13 |
| Stainless steel | 18.7 | 3.71 | 5 |
| Nickle based Super alloy | 12.1 | 3.62 | 3 |
| Titanium Alloy | 9.9 | 2.99 | 3 |
| Nickle chromium Alloy | 11.6 | 3.69 | 3 |
| CFRP epoxy matrix | 1.82 | 1.5 | 1 |

Mechanical loss coefficient (tanδ) and Young’s Modulus and Density: The mechanical loss-coefficient also called damping factor measures the degree to which a material dissipates vibrational energy.

If a material subjected to elastic loading of σmax ,then it stores elastic energy per unit volume U:

U=0σmax σ dε= 12 σmax2E

If the material is then unloaded, it dissipates an energy:

ΔU=∮ σdε

The loss coefficient is

η=ΔU2πU

The value of η usually depends on the frequency of cycling. Damping can also measure by

- specific damping capacity

D=ΔU/U

- The logarithmic decrement,

∆ =ln⁡(x2x1)

Where x1 and x2 are successive amplitudes of natural vibrations.

- The phase-lag, δ, between stress and strain, and the " Q "-factor or resonance factor, Q.

When damping is small (η<0.01) all of the above related by:

η=D2π=Δπ=tanδ=1Q

For large damping above relations are not valid. Damping for case of shaft design is considered small while for case of spring Design it considered large.

Δ= 2πη1-η2
for  η<0.01          1-η2 ≈1

Δ =πtanδ=2πη1-η2Δ =πtanδ=2πη=2πCCc=2πC2km =2πC2mgδ⋅m=2πC2mgδ =C11mWL348EI=C21m 1E 1r2Δ =C2mE πLρmΔ =C3ρEm2=πtanδm2 =ρE tanδ  minimise

P9= E tanδρmaximise

P9=Eρtanδmaximize

| Material | E | ρtanδ | Eρtanδmaximize |
| --- | --- | --- | --- |
| CFRP epoxy matrix | 102 | 0.721 | 14.0076 |
| Stainless steel | 200 | 1.27 | 11.1355 |
| Age hardning wrought Al Alloy | 71.9 | 2.5 | 3.3918 |
| Titanium Alloy | 115 | 4.6 | 2.3313 |
| Nickle chromium Alloy | 210 | 9.39 | 1.5433 |
| Medium carbon steel | 210 | 13.2 | 1.0978 |
| High Carbon Steel | 207 | 14.5 | 0.9922 |
| Nickle based Super alloy | 210 | 22.1 | 0.6557 |
| Tungsten Alloys | 339 | 102 | 0.1805 |
| Carbon fiber (HM Carbon) | 380 | 129 | 0.1511 |

Relative cost and material class:

Relative cost=cost of materialkg*density of materialcost of mild steelkg*density of mild steel

Modulus and relative cost:

- Young’s Modulus and Density:

Bending Equation:

MI=σY=ER

Bending Stiffness:

Sb=EIL
Combining above two equations
Sb=EΓL=Eπγ44L
r=mπLρ
Sb=Eπ4LmπLρ2
m2=4SbπL2ρ2E
m=4SbπL2ρE12

Thus, performance index given by
P2=(E1/2ρ)Maximize

Dividing by relative cost:

P11=(E1/2Cmρ)Maximize

Conclusion:

The most important conclusion drawn is that a specific class of material is suitable for particular application and since the function of the component is same hence by changing the performance  index there is only change in  sequence of that class of material only and no  new material going to add while chan ging the performance index.

Below is given a class of material for the shaft.

Carbon fiber (HS carbon)

Carbon fiber (Hm carbon)

Glass fiber ( C - glass)

CFRP (epoxy matrix)

Titanium alloys

High carbon steel

Medium carbon steel

Stainless steel

Nickle alloy

Based on different application of shaft above materials are used at different places

Reference:

[1] class notes

[2] Ces Edupack software

[3] https://en.wikipedia.org/wiki/Shaft_(mechanical_engineering)

[4]Material selection in mechanical design book by Ashby